Adding Custom IP to the System

Introduction

This lab guides you through the process of creating and adding a custom peripheral to a processor system by using the Vivado IP Packager. You will create an AXI4Lite interface peripheral.

# Objectives

After completing this lab, you will be able to:

Use the IP Packager feature of Vivado to create a custom peripheral

Modify the functionality of the IP

Add the custom peripheral to your design

Add pin location constraints

Add block memory to the system

Procedure

This lab is separated into steps that consist of general overview statements that provide information on the detailed instructions that follow. Follow these detailed instructions to progress through the lab.

This lab comprises 4 primary steps: You will use a peripheral template to create a peripheral, Package the IP using IP Packager, import, add and connect the IP in the design, and add the Block RAM (BRAM) Memory.

Design Description

You will extend the Lab 2 hardware design by creating and adding an AXI peripheral (refer to LED_IP in Figure 1) to the system, and connecting it to the LEDs on the Zynq board you are using.  You will use the IP Packager to generate the custom IP.    Next, you will connect the peripheral to the system and add pin location constraints to connect the LED display controller peripheral to the on-board LED display.  Finally, you will add BRAM Controller and BRAM before generating the bitstream.

Figure 1. Design Updated from Previous Lab

General Flow for this Lab

Step 1: Create Custom IP using

Peripheral Template

Step 2: Package Peripheral using IP Packager

Step 3:

Set the Project Settings

Step 4:

Add the IP. BRAM, and Constraints

In the instructions below;

{sources} refers to: C:\xup\embedded\2015_2_zynq_sources

{labs} refers to : C:\xup\embedded\2015_2_zynq_labs

{labsolutions} 	for the ZedBoard refers to: 	C:\xup\embedded\2015_2_zedboard_labsolution

or for the Zybo refers to: 	C:\xup\embedded\2015_2_zybo_labsolution

Create a Custom IP using the Create and Package IP Wizard	Step 1

Use the provided axi_lite slave peripheral template and the custom IP source code to create a custom IP.

Open Vivado by selecting Start > All Programs > Xilinx Design Tools > Vivado 2015.2 > Vivado 2015.2

Click Manage IP and select New IP Location and click Next in the New IP Location window

Select Verilog as the Target Language, Mixed as the Simulator language, and for IP location, type {labs}\led_ip and click Finish (leave other settings as defaults and click OK if prompted to create the directory)

Figure 2. New IP Location form

A Virtex 7 part is chosen for this project, but later compatibility for other devices will be added to the packaged IP.

Run the Create and Package IP Wizard

Select Tools > Create and Package IP

In the window, click Next

Select Create a new AXI4 peripheral, and click Next

Fill in the details for the IP

- Name: led_ip

- Display Name: led_ip_v1_0

- (Fill in a description, Vendor Name, and URL)

Figure 3. Updating Peripheral Details form

Click Next

Change the Name of the interface to S_AXI

Leave the other settings as default and click Next (Lite interface, Slave mode, Data Width 32, Registers 4)

Figure 4. Naming the AXI interface

Select Edit IP and click Finish (a new Vivado Project will open)

Create an interface to the LEDs

In the sources panel, double-click the led_ip_v1_0.v file.

This file contains the HDL code for the interface(s) selected above. The top level file contains a module which implements the AXI interfacing logic, and an example design to write to and read from the number of registers specified above. This template can be used as a basis for creating custom IP. A new parameterized output port to the LEDs will be created at the top level of the design, and the AXI write data in the sub-module will be connected back up to the external LED port.

Scroll down to line 7 where a user parameters space is provided.

Add the line:

- parameter integer LED_WIDTH             = 8,

Go to line 18 and add the line:

- output wire [LED_WIDTH-1:0]      LED,

(Notice the extra comma needed at the end of each line)

Figure 5. Adding users parameter, and port definition

Insert the following at line ~48:

- .LED_WIDTH(LED_WIDTH),

Insert the following at line ~52:

- .LED(LED),

Figure 6. Adding port connection with a lower-level module

Save the file by selecting File > Save File

Expand led_ip_v1_0 in the sources view if necessary, and open led_ip_v1_0_S_AXI.v

Add the LED parameter and port to this file too, at lines 7 and 18

Figure 7. Declaring users port in the lower-level module for the Zybo

Scroll down to ~line 400 and insert the following code to instantiate the user logic for the LED IP

(This code can be typed directly, or copied from the user_logic_instantiation.txt file in the lab3 source folder.)

Figure 8. Instantiating lower-level user module

Check all the signals that are being connected and where they originate.

Save the file by selecting File > Save File

Click on the Add Sources in the Flow Navigator pane, select Add or Create Design Sources, click Next, then click the Green Plus then Add Files…, browse to {sources}\lab3, select the lab3_user_logic.v file and click OK, and then click Finish to add the file.

Check the contents of this file to understand the logic that is being implemented.  Notice the formed hierarchy.

Click Run Synthesis and Save if prompted. (This is to check the design synthesizes correctly before packaging the IP. If this was your own design, you would  simulate it and verify functionality before proceeding)

Check the Messages tab for any errors and correct if necessary before moving to the next step

When Synthesis completes successfully, click Cancel.

Package the IP

Click on the Package IP – led_ip tab

Figure 9. Package IP

For the IP to appear in the IP catalog in particular categories, the IP must be configured to be part of those categories. To change which categories the IP will appear in the IP catalog click Green Plus in the Categories section. This opens the Choose IP Categories window

For the purpose of this exercise, uncheck the AXI Peripheral box and check the Basic Elements and click OK.

Figure 10. Specify the category for IP Packager IP

Select Compatibility. This shows the different Xilinx FPGA Families that the IP supports. The value is inherited from the device selected for the project.

Click the Green Plus then Add Family Explicitly… from the menu.

Select the Zynq family as we will be using this IP on the Zybo and Zedboard, and click OK.

You can also customize the address space and add memory address space using the IP Addressing and Memory category.  We won’t make any changes.

Click on File Groups and click Merge changes from File Groups Wizard

Figure 11. Updating the file group

This is to update the IP Packager with the changes that were made to the IP and the lab3_user_logic_*.v file that was added to the project.

Expand Verilog Synthesis and notice lab3_user_logic.v has been included

Click on Customization Parameters and again Merge changes from Customization Parameters Wizard

Notice that the Ports and Interfaces view now shows the user created LED port

Figure 12. User parameter and port

Select Customization Parameters, expand Hidden Parameters, right-click on LED_WIDTH, and select Import IP Parameters… and click OK.

Select Customization GUI and notice that the Led Width is visible.

Figure 13. User customization parameter

Select Review and Package, and notice the path where the IP will be created.

Click Package IP. Click Yes and the project will close when complete.

In the original Vivado window click File > Close Project

Modify the Project Settings	Step 3

Open the previous project, or use the lab2 project from the labsolutions directory, and save the project as lab3. Set Project Settings to point to the created IP repository.

Start the Vivado if necessary and open either the lab2 project you created in the previous lab or the lab2 project in the labsolution directory

Select File > Save Project As … to open the Save Project As dialog box. Enter lab3 as the project name.  Make sure that the Create Project Subdirectory option is checked, the project directory path is {labs}\ and click OK.

This will create the lab3 directory and save the project and associated directory with lab3 name.

Click Project Settings in the Flow Navigator pane.

Select IP in the left pane of the Project Settings form.

Click on the Green Plus button, browse to {labs}\led_ip and click Select.

The led_ip_v1.0 IP will appear the IP in the Selected Repository window.

Figure 14. Specify IP Repository

Click OK.

Add the Custom IP, BRAM, and the Constraints 	Step 4

Add led_ip to the design and connect to the AXI4Lite interconnect in the IPI.  Make internal and external port connections.   Establish the LED port as external FPGA pins.

Click Open Block Design under IP Integrator in the Flow Navigator pane

Click the Add IP icon  and search for led_ip_v1.0 in the catalog by typing “led” in the search field.

Figure 15. Searching for led_ip in the IP Catalog

Double-click led_ip_v1_0 to add the core to the design.

Select the IP in the block diagram and change the instance name to led_ip in the properties view.

Double click the block to open the configuration properties

For the ZedBoard, leave the Led Width set to 8, or for the Zybo, set the width to 4.

Click OK.

Figure 16. Configure the LED IP LED_WIDTH

Click on Run Connection Automation, select /led_ip/S_AXI and click OK to automatically make the connection from the AXI Interconnect to the IP.

- Click the regenerate button () to redraw the diagram.

Select the LED port on the led_ip instance (by clicking on its pin), right-click and select Make External.

Figure 17. LED external port added and connected

Select the Address Editor tab and verify that an address has been assigned to led_ip.

Figure 18. Address assigned for led_ip

Add BRAM to the design

In the Block Diagram, click the Add IP icon  and search for BRAM and add one instance of the AXI BRAM Controller

Run Connection Automation on axi_bram_ctrl_0/S_AXI and click OK when prompted to connect it to the M_AXI_GP0 Master.

Double click on the block to customize it and change the number of BRAM interfaces to 1 and click OK.

Notice that the AXI Protocol being used is AXI4 instead of AXI4Lite since BRAM can provide higher bandwidth and the controller can support burst transactions.

Figure 19. Customize BRAM controller

Click on Run Connection Automation to add and connect a Block Memory Generator by selecting axi_bram_ctrl_0/BRAM_PORTA and click OK (This could be added manually)

Validate the design to ensure there are no errors (F6), and click the regenerate button () to redraw the diagram.

- The design should look similar to the figure below.

Figure 20. Completed Block Diagram

In the Address editor, notice the Range of the axi_bram_ctrl_0 is 8K. We will leave it at that.

Figure 21. Adjusting memory size

Press F6 to validate the design one last time.

Add the provided lab3_*.xdc constraints file.

Click Add Sources in the Flow Navigator pane, select Add or Create Constraints, and click Next.

Click the Green Plus button, and then Add Files…, browse to the {sources}\lab3 folder, select lab3_zed.xdc for the ZedBoard, or lab3_Zybo.xdc  for the Zybo

Click Finish to add the file.

Expand Constraints folder in the Sources pane, and double click the lab3_*.xdc file entry to see its content. This file contains the pin locations and IO standards for the LEDs on the Zynq board. This information can usually be found in the manufacturer’s datasheet for the board.

Right click on system.bd and select Generate output products

Click on Generate Bitstream and click Yes if prompted to save the Block Diagram, and click Yes again if prompted to launch synthesis and implementation. Click Cancel when prompted to Open the Implemented Design

Conclusion

Vivado IP packager was used to import a custom IP block into the IP library. The IP block was then added to the system. Connection automation was run where available to speed up the design of the system by allowing Vivado to automatically make connections between IP. An additional BRAM was added to the design. Finally, pin location constraints were added to the design.